Experiment 3

# Setting Up Hybrid Cloud Connectivity Using Cisco Packet Tracer (Simulation)

AIM:

To simulate a secure hybrid cloud connection between an on-premises data center and a cloud network using routers, switches, subnets, and routing in Cisco Packet Tracer

PROCEDURE:

Step 1: Open Cisco Networking Academy

Go to: https://www.netacad.com

# Step 2: Create a Free Account or Sign in with your Google Account

- Click Create an account (or Sign up).

- Enter:

o First Name o Last Name o Email Address o Password

- Verify your email if prompted.

- Sign in.

# Step 3: Enroll in the Free Packet Tracer Course

# Step 4: Download Packet Tracer

After enrolling:

- Go to your NetAcad Dashboard.

- Open the Getting Started with Cisco Packet Tracer course.

- Navigate to the Resources or Downloads section. 4. Download the version for your operating system: o Windows 64-bit (recommended for most PCs) o macOS o Linux

# Step 5: Install Packet Tracer

- Open the downloaded installer (.exe on Windows).

- Click Next.

- Accept the license agreement.

- Click Next until installation completes.

- Click Finish.

# Step 6: Start Packet Tracer

Open Cisco Packet Tracer from the Start Menu.

Sign in with your Cisco NetAcad account or sign in with your google account.

# Step 7: Place Two Routers

At the bottom-left of Packet Tracer, you'll see several icons.

- Click Network Devices (the router icon).

- Click Routers.

- Choose 2911 (or 1941 if 2911 isn't available).

- Click once in the blank workspace to place the first router and second one

# Step 8: Place Two Switches

- At the bottom-left, click Network Devices.

- Click Switches.

- Select 2960 (for example, 2960-24TT).

- Place one switch below Router0.

- Place another switch below Router1.

# Step 9: Place One PC and One Server

- At the bottom-left, click End Devices (computer icon).

- Select PC.

- Place it to the left of Switch0.

- Select Server.

- Place it to the right of Switch1.

| Step 10: Connect the Devices At the bottom-left, click the Connections icon )( lightning bolt). Choose Copper Straight-Through (usually the first black cable). Connection 1: PC to Switch0 - Click PC. - Select FastEthernet0. - Click Switch0. - Select FastEthernet0/1 (or any free FastEthernet port). Connection 2: Switch0 to Router0 Using the same Copper Straight-Through cable: - Click Switch0. - Select FastEthernet0/2. - Click Router0. - Select GigabitEthernet0/0. Connection 3: Server to Switch1 Again use Copper Straight-Through: - Click Server. - Select FastEthernet0. - Click Switch1. - Select FastEthernet0/1. Connection 4: Switch1 to Router1 Again use Copper Straight-Through: - Click Switch1. - Select FastEthernet0/2. - Click Router1. - Select GigabitEthernet0/0. | 231801026 |
| --- | --- |

## Step 10.1: Connect Router0 to Router1

Now connect the two routers. Option A (Recommended)

Use Copper Cross-Over.

- Click the Connections icon.

- Select Copper Cross-Over.

- Click Router0.

- Select GigabitEthernet0/1.

- Click Router1.

- Select GigabitEthernet0/1.

Your topology should now look like this:

# Step 10.2: Click the Router

In your workspace, left-click Router0 once.

- Click the CLI tab.

- If it asks:

Continue with configuration dialog? [yes/no]:

Type: no Press Enter. When you see: Router>

# Step 10.3: Configure the LAN Interface (Connected to Switch0)

At the Router(config)# prompt, type these commands one at a time, pressing Enter after each line.

interface 	gigabitEthernet0/0

You should see: Router(config-if)#

Now type:

ip address 192.168.1.1 255.255.255.0

Press Enter. Now type: no shutdown

You should see a message similar to: Now

## Step 10.4: Configure the Router-to-Router Interface

Now type: interface 	gigabitEthernet0/1

Then:

ip address 10.10.10.1 255.255.255.252

Then:

no shutdown

Again, wait for the interface to come up.

Finally, type:

exit

You should now be back at:

Router(config)#

## Step 10.5: Verify the Configuration

Type:

do show ip interface brief

Since you're in configuration mode, use do before the show command.

You should see something similar to:

## Next Step: Configure Router1

Click Router1 → CLI.

If it says:

Press RETURN to get started!

Press Enter.

If it asks:

Continue with configuration dialog? [yes/no]:

Type: no

Then enter these commands one by one: enable configure terminal interface gigabitEthernet0/0

ip address 10.0.0.1 255.255.255.0 no shutdown

exit interface gigabitEthernet0/1 ip address

## Step 10.6: Configure the PC

- Click PC.

- Select Desktop.

- Click IP Configuration.

## Step 10.7: Configure the Server

- Click Server.

- Select Desktop.

- Click IP Configuration.

# Step 11: Configure Static Routes

## A. Configure Static Route on Router0

- Click Router0.

- Go to the CLI tab.

- If you're at Router#, type:

configure terminal

Now enter: ip route 10.0.0.0 255.255.255.0 10.10.10.2

Exit configuration mode: exit

Save the configuration: copy running-config startup-config

When prompted: Destination filename [startup-config]? Just press Enter.

## B. Configure Static Route on Router1

- Click Router1.

- Open the CLI.

At the Router# prompt, type: configure terminal

Now enter: ip route 192.168.1.0 255.255.255.0 10.10.10.1

Exit: exit

Save the configuration: copy running-config startup-config When prompted for the filename, simply press Enter.

## C. Verify the Route

Type: show ip route

You should see a line similar to: S 192.168.1.0/24 [1/0] via 10.10.10.1 The S indicates that the static route has been added successfully.

## Now let's verify Router0

On Router0, at the Router# prompt, type: show ip route You should see a line like: S 10.0.0.0/24 [1/0] via 10.10.10.2

If you don't see it, add the route:

configure terminal ip route 10.0.0.0 255.255.255.0 10.10.10.2 end copy running-config startup-config

Press Enter when it asks for the destination filename.

# Step 12: Test Connectivity

## From the PC

- Click PC.

- Go to Desktop → Command Prompt.

- Type: ping 10.0.0.10

- Type one more time: ping 10.0.0.10

Experiment Successfully Completed!

You have successfully simulated Hybrid Cloud Connectivity.

# Architecture Diagram of Simulation

Result:

A hybrid cloud network was successfully simulated using Cisco Packet Tracer. The on- premises network and cloud network were interconnected through routers configured with static routing. Communication between the PC and the cloud server was verified successfully using the ping command, demonstrating end-to-end connectivity.